PRODUCT ENGINEERING - RELIABILITY

FMEA Ratings and Action Priority Reference

AIAG-VDA method (2019)  |  Detection rating map and Action Priority table

DOC FDS-DFMEA-RATE Rev A

This reference gives the rating method for the Design FMEA. Severity and Occurrence values are supplied per failure mode in the register. Detection is not supplied; derive it from the current detection control named in the register using the Detection map below. Then determine Action Priority from the Action Priority table. Action Priority replaces RPN; do not multiply S, O, and D.

1   Detection Rating from Current Control

Look up the failure mode's current detection control in the register, then read its Detection rating here. Lower detection numbers mean better detection.

| Current detection control | Detection rating (D) |
| --- | --- |
| 100% automated with error-proofing | 2 |
| Design validation + in-line PPAP | 3 |
| In-line automated leak test | 4 |
| Pressure decay test (in-line) | 4 |
| Bench durability test | 5 |
| Functional end-of-line test | 6 |
| Visual inspection only | 8 |
| None / not detectable | 10 |

2   Action Priority Table

Action Priority is High, Medium, or Low, determined by the combination of Severity, Occurrence, and Detection. Find the Severity band, then the Occurrence band, then read the Detection column. This is a lookup, not a calculation.

| Severity | Occurrence | D 1-4 | D 5-7 | D 8-10 |
| --- | --- | --- | --- | --- |
| 9-10 | 6-10 | H | H | H |
| 9-10 | 4-5 | H | H | H |
| 9-10 | 2-3 | H | H | H |
| 9-10 | 1 | M | M | H |
| 7-8 | 6-10 | H | H | H |
| 7-8 | 4-5 | M | H | H |
| 7-8 | 2-3 | L | M | H |
| 7-8 | 1 | L | M | M |
| 4-6 | 6-10 | M | H | H |
| 4-6 | 4-5 | L | M | H |
| 4-6 | 2-3 | L | L | M |
| 4-6 | 1 | L | L | L |
| 2-3 | 6-10 | L | M | M |
| 2-3 | 4-5 | L | L | M |
| 2-3 | 2-3 | L | L | L |
| 2-3 | 1 | L | L | L |
| 1 | 6-10 | L | L | L |
| 1 | 4-5 | L | L | L |
| 1 | 2-3 | L | L | L |
| 1 | 1 | L | L | L |

Severity 1 is Low for every Occurrence and Detection.

3   Action Rule

Action Priority High: action is required. Enter Yes in the action column.

Action Priority Medium: review and justify. Enter Review.

Action Priority Low: acceptable as-is. Enter No.

4   Ranking

Rank the completed failure modes by Action Priority (High, then Medium, then Low), and within each priority by descending Severity, then Occurrence, then Detection.