PRODUCT ENGINEERING - RELIABILITY

Fuel Delivery System - Design FMEA Scope

Program FX-mid gasoline PFI  |  Subsystem definition and functions

DOC FDS-DFMEA-SCOPE Rev A

This document defines the fuel delivery subsystem for the Design FMEA: its items, their functions, and the intended performance. It does not assign risk ratings or list failure modes. Ratings method and thresholds are in the FMEA Ratings and Action Priority Reference (FDS-DFMEA-RATE). The failure-mode working list with severity, occurrence, and current detection controls is in the failure-mode register (FDS-DFMEA-MODES.xlsx). The DFMEA is completed by scoring that register against the reference.

1   Subsystem Boundary

The fuel delivery subsystem carries gasoline from the tank to the injectors at regulated pressure and meters it into the intake port on command. In scope: fuel pump, filter, lines and quick connectors, rail, pressure regulator and its diaphragm, injectors and their seals, and tank sealing at the pump flange. Out of scope: tank structure, evap canister, engine control strategy.

2   Items and Functions

| Item | Primary function | Performance requirement |
| --- | --- | --- |
| Fuel Pump | Deliver rated flow and pressure | Maintain rail pressure across the flow range |
| Fuel Filter | Filter particulate | Protect injectors, hold flow across service life |
| Fuel Rail | Contain pressurized fuel, distribute to injectors | No external leak at rated cycling |
| Quick Connector | Join fuel lines | Retain and seal under vibration |
| Pressure Regulator | Regulate rail pressure | Hold set pressure across load |
| Regulator Diaphragm | Seal reference chamber | No rupture across durability |
| Injector | Meter fuel per command | Deliver commanded quantity, seal at seat |
| Tank Seal | Contain fuel vapor and liquid at pump flange | No weep or evap leak |

3   Failure Consequences of Interest

Failure effects range from driveability and emissions concerns to safety. Any external fuel leak or unseated connection is treated as a fire-risk safety effect and carries the highest severity. The severity value for each failure mode is provided in the register; do not re-rate severity here.