GN&C / prop notes before you size the delta-V budget

from: Priya N. (GN&C)  to: mission analysis  re: HTS-1 pre-ship prop budget  2026-06-21

Before you build the prop budget, the working maneuver sheet (HTS-GNC-021) leaves the apogee and disposal burns as DERIVE on purpose, you have to compute those from the orbit parameters tab. Don't go hunting for a delta-V number that isn't there; size it yourself per the ground rules. A few things to get right:

Do the apogee burn as a SINGLE combined maneuver, circularize and take out all 27 degrees of inclination at once. Don't split it into a separate circularization and a separate plane change and add them, that overstates the delta-V by a couple hundred m/s. Law of cosines on the apogee and GEO velocity vectors.

Isp: do NOT use the 322 nameplate. The flight engine came back from acceptance hot-fire at 318.5 s delivered. That's the number. The lower Isp is part of why margin is tight, so don't paper over it.

M1 is dead. We descoped the phasing burn, the customer agreed to drop us closer to the service longitude. Pull M1 out of the budget entirely.

M3 (the GEO trim) is in feet per second on that sheet, 115 ft/s. Somebody pulled it off the old GN&C memo. Convert it to metric before you use it.

Sequencing matters for the mass: the payload separates right after the apogee insertion (circularization). So the trim and disposal burns are flown on the light stage, payload gone. Don't carry the 500 kg through them.

Propellant: the plan said 800 kg main but we did NOT load that. We stopped at the flowmeter target. The actual load is on the tanking record (MMH + NTO). Use the as-loaded number, not 800.

The GHe pressurant on the tanking record is loaded through the GSE, it's not flight propellant, leave it out of the prop mass.

Hydrazine came in at 9.2 loaded against a 7.5 budget, so RCS is fine, but show it.

What I need is the pre-ship delta-V and propellant budget: the derived maneuver delta-V with losses, the rocket-equation prop burn per maneuver accounting for payload sep, usable vs required main prop, and where our margin lands against the 5% gate. If we're short, tell me how much to top off. One workbook is fine, just make it clean enough to drop into the pre-ship review.

- Priya