HTS-1 TRANSFER STAGE
Propulsion & Delta-V Budget Ground Rules

Doc HTS-PROP-007 Rev B  |  Effective 2026-05-20  |  INTERNAL / Helios Space

1.  Vehicle and mission

HTS-1 is a storable bipropellant (MMH/NTO) transfer stage that delivers a customer payload from a GTO drop-off to GEO, separates the payload after circularization, and then disposes itself to a graveyard orbit. Reaction control is monopropellant hydrazine (N2H4).

2.  Delta-V derivation (required)

The apogee insertion and disposal delta-V are NOT pre-tabulated; derive them from the orbit parameters in the maneuver plan workbook using two-body (patched-conic) methods:

a) Combined apogee insertion (M2): at GTO apogee, perform the circularization burn to GEO and remove the full GTO inclination in a single combined maneuver. Use the vis-viva equation for the GTO apogee velocity and the circular GEO velocity, then the law of cosines with the plane-change angle: dV = sqrt(Va^2 + Vc^2 - 2*Va*Vc*cos(i)).

b) Disposal (M4): a two-burn Hohmann transfer from GEO to a circular orbit 300 km above GEO; sum the two impulsive burns.

c) Phasing (M1) and GEO trim (M3) are given directly in the maneuver plan.

3.  Fixed masses (per ICD)

| Item | Value |
| --- | --- |
| Stage dry mass | 320.0 kg |
| Delivered payload | 500.0 kg |
| Payload separation | Immediately after circularization (end of the apogee insertion burn) |

4.  Engine performance

Main engine specific impulse (Isp), nameplate/spec value: 322.0 s. Where an engine acceptance (hot-fire) Isp is available for the as-built engine, the delivered acceptance value supersedes the nameplate for budgeting. Use g0 = 9.80665 m/s^2 and the ideal rocket equation (dV = Isp * g0 * ln(m0/mf)). Nominal mixture ratio (ox/fuel) is 1.65.

5.  Losses and residuals

Apogee (main) burns carry a 1.5% finite-burn and steering loss applied to ideal delta-V. On-station and disposal burns are impulsive (no loss factor). Main propellant residual (unusable holdup + trapped) is 2.0% of loaded main propellant; only usable propellant counts toward the budget.

6.  Propellant accounting

Main propellant loaded shall be taken from the as-loaded tanking record (MMH + NTO), not the planning target. Pressurant loaded via ground support equipment (GHe) is not flight propellant mass. RCS hydrazine budget over the mission is 7.5 kg.

7.  Margin policy

At the pre-ship review, usable main propellant margin shall be at least 5%, defined as (usable loaded - required) / required. Margin below 5% requires a top-off to tank capacity (main tanks are sized for an 800 kg planning load) or a documented waiver.